# Program IPO & Requirements Decomposition

Engineer: ___Prabhdeep Singh__________

## Section #1 - Program IPO (Input, Processing, Output

##### Program Task: Explain the task that the program is to perform.

The program's task is to read a file provided by the user and display its content on the console, showing 24 lines at a time. After displaying 24 lines, it prompts the user to press "Enter" to continue to the next set of lines.

##### Program Input: Explain any input that is needed by the program, and how it will be provided (file or user input).

- File name: A string input provided by the user through the console.

- File content: The content of the file specified by the file name.

##### Program Output: Explain the program results and output.

The output is the content of the file displayed on the console, with line numbers prefixed to each line. If the file does not exist or cannot be opened, it displays an error message.

## Section #2 - Design Considerations

##### Explain the overall design and include any specific design considerations and data types for this program.

The overall design is a simple file reader utility. It uses a while loop to read each line of the file until there is no content left. It uses the <fstream> library to handle file input operations and the <iomanip> library to manage output formatting.

- Data Types:

  - fstream: to handle file operations

  - string: to store file name and lines read from the file

  - int: to store line numbers

## Section #3 - Order of operations/flow of control

##### Explain the order of operations for the solution at a high level, step-by-step in pseudo-code.

- Declare necessary variables and libraries.

- Prompt the user to enter the file name.

- Open the file with the specified name in read mode.

- Check if the file has been opened successfully; if not, display an error message and exit the program.

- Use a while loop to read each line from the file until the end of the file is reached:

  - Display the line number and the line content in the console.

  - If 24 lines have been displayed, prompt the user to press "Enter" to continue.

- Close the file after reading all lines.

- Exit the program.

## Section #4 - Program Requirements

##### Explain what the program requires to complete the task.

- Libraries Needed:

  - <iostream>: for input-output streams

  - <fstream>: for file handling

  - <iomanip>: for output formatting

- Variables:

  - datafile: to handle file operations

  - fileName: to store the file name input by the user

  - line: to store each line read from the file

  - lineNumber: to store the current line number being read

- Special Output Formatting:

  - setw: to set the width of the next input/output field

  - Line numbering with each line of file content

## Section #5 – Functions (required for each function in the program)

#### Function IPO (Input, Processing, Output): complete an IPO section for each function in the program.

Function name: main

Declared as: int

Task: To read a file's content and display it on the console with line numbers, showing 24 lines at a time.

External requirements: A file with the specified name should exist in the file system.

Internal requirements: Proper initialization of variables and successful file opening for reading.

Internal operation: Reading lines from the file and displaying them on the console with proper formatting and line numbering.

Return Value:

- 0: if the program executes successfully

- 1: if the file opening fails